STANDARD FUNCTIONS- Completed

- BLE peripheral name: ESP32-JOE

- Service UUID (Nordic UART / NUS): 6e400001-b5a3-f393-e0a9-e50e24dcca9e

- App steps (connect):

  - Scan for a BLE device named ESP32-JOE (or advertising the NUS service UUID above)

  - Connect

  - Discover services/characteristics

  - Find these characteristics under the NUS service:

    - RX (WRITE to ESP32): 6e400002-b5a3-f393-e0a9-e50e24dcca9e

    - TX (NOTIFY from ESP32): 6e400003-b5a3-f393-e0a9-e50e24dcca9e

To send a Standard Wave command (STD):

  - Do a GATT Write to the RX characteristic (6e400002-...)

  - Payload is 17 bytes total, little-endian for multi-byte numbers:

- STD packet format (17 bytes):

  - Byte 0: 'S' (0x53)

  - Byte 1: shape (u8)

    - 0 = sine

    - 1 = square

    - 2 = triangle

    - 3 = sinc

  - Bytes 2–5: freq_mHz (u32, little-endian)

    - freq in milli-Hz = Hz * 1000

  - Bytes 6–9: amp_mVpp (u32, little-endian)

    - amplitude in mV peak-to-peak = Vpp * 1000

  - Bytes 10–13: offset_mV (s32, little-endian)

    - DC offset in mV = V * 1000 (signed)

  - Bytes 14–15: phase_deg (s16, little-endian)

    - phase in degrees (signed)

  - Byte 16: channel (u8)

    - 0 = channel A

    - 1 = channel B

- Example values (so they can sanity check):

  - 1 kHz → freq_mHz = 1,000,000

  - 2.0 Vpp → amp_mVpp = 2000

  - +0.5 V offset → offset_mV = 500

  - phase 90° → phase_deg = 90

  - channel A → channel = 0

ARBITRARY FUNCTIONS- SCRAPED

“Interpolation” of points to reduce transmission time.

Here’s a quick write-up you can paste to the app person. It’s written like a mini “protocol spec”.

## ESP32 BLE Connection + Arbitrary Waveform Upload (Protocol)

### 1) BLE service + characteristics

The ESP32 advertises as ESP32-JOE and uses the Nordic UART Service (NUS).

Discover:

- Service UUID: 6e400001-b5a3-f393-e0a9-e50e24dcca9e

- RX (Write) characteristic UUID: 6e400002-b5a3-f393-e0a9-e50e24dcca9e ← app writes packets here

- TX (Notify) characteristic UUID: 6e400003-b5a3-f393-e0a9-e50e24dcca9e (optional; not required for upload)

Connection steps:

- Scan for ESP32-JOE

- Connect

- Discover NUS service

- Write all commands to the RX characteristic

### 2) Arbitrary waveform upload overview

Arbitrary waveform upload is 3 phases:

- META: start upload + set channel + set frequency

- DATA: stream 16-bit samples in chunks(Edit this so app sends all 64k points)

- END: commit (only succeeds if exactly 1024 samples received)

Total samples required: 1024 (exactly)

All multi-byte integers are Little Endian (LE).

### 3) Packets

#### A) META packet (start new ARB upload)

Format:
'A' + channel(1 byte) + freq_mHz(u32 LE)

- Byte 0: ASCII 'A' = 0x41

- Byte 1: channel:

  - 0 = Channel A

  - 1 = Channel B

- Bytes 2–5: freq_mHz as uint32 LE

  - Frequency is in milli-Hz (Hz * 1000)

Example: Channel A, 1000 Hz
freq_mHz = 1,000,000
Payload: [0x41, 0x00, 0x40, 0x42, 0x0F, 0x00]

Sending META resets/clears the ARB buffer on the MCU.

#### B) DATA packet (send samples)

Format:
'D' + count(u16 LE) + samples[count] (each sample u16 LE)

- Byte 0: ASCII 'D' = 0x44

- Bytes 1–2: count = number of samples in this packet (uint16 LE)

- Remaining bytes: count samples, each a uint16 LE DAC code

Repeat DATA packets until the MCU has received 1024 total samples.

#### C) END packet (commit)

Format:
'Z'

- Single byte: ASCII 'Z' = 0x5A

Commit rules:

- MCU only commits if META was received and exactly 1024 samples have been received total.

- If not 1024, it rejects the upload.

### 4) Sample encoding (what the 16-bit numbers mean)

Each sample is a raw uint16 DAC code: 0 … 65535.

If your app generates values in volts and the hardware range is ±5 V, map like this (clamp to range):

- code = round((v + 5.0) * (65535 / 10.0))

- v = −5.0 → 0

- v = +5.0 → 65535

(If the app already generates 0..65535 codes, send directly.)

### 5) BLE payload sizing / chunk size

DATA packet size is: 3 + 2*count bytes.

Pick count so the whole DATA packet fits in the BLE write payload (depends on MTU).

- If MTU is default (~20 payload bytes), max count ≈ 8 samples per packet.

- If MTU is negotiated larger (recommended), you can send bigger chunks (ex: 60 samples → 123 bytes).

Recommendation:

- Request a higher MTU if supported and stream DATA packets sequentially (write-with-response preferred for reliability).

### 6) Minimal send sequence summary

- Connect to BLE device ESP32-JOE

- Write META: 'A' + channel + freq_mHz(LE u32)

- Send DATA packets until 1024 samples total are sent

- Write END: 'Z'

# Arbitrary Waveform Upload

### BLE Packet Flow – Explained (with Real Example)

This document explains why multiple BLE packets are sent when uploading an arbitrary waveform and what each packet means on the MCU side.

## Overview

When sending an arbitrary waveform to the MCU:

- The waveform always contains exactly 1024 samples

- Samples are sent in chunks to fit BLE packet size limits

- The upload happens in three phases:

  - META (start upload)

  - DATA (send waveform samples)

  - END (commit waveform)

## Packet Types Summary

| Packet | Byte | Purpose |
| --- | --- | --- |
| META | 'A' (0x41) | Start ARB upload, set channel & frequency |
| DATA | 'D' (0x44) | Send waveform samples |
| END | 'Z' (0x5A) | Commit waveform (only if 1024 samples received) |

Any packet that is not the standard 17-byte AWG packet ('S') is logged by the MCU as NON-STANDARD PACKET — this is expected.

## Real Example From MCU Log

RX write [123 bytes]

RX write [11 bytes]

RX write [1 bytes]

This is correct and expected behavior.

## 1️⃣ First Packet – DATA (123 bytes)

44 3c 00 ...

### What it means:

- 0x44 → 'D' = DATA packet

- 0x3c 0x00 → 60 samples (little-endian)

- Remaining bytes → 60 × 2 = 120 bytes of sample data

### Why 123 bytes total?

1 byte  → 'D'

2 bytes → sample count

120 bytes → waveform samples

----------------------------

123 bytes total

✔ This is the main waveform chunk
✔ Size chosen to fit BLE MTU safely

## 2️⃣ Second Packet – DATA (11 bytes)

44 04 00 45 67 45 67 45 67 45 67

### What it means:

- 'D' → DATA packet

- 0x04 0x00 → 4 samples

- 4 × 2 = 8 bytes of data

### Why only 4 samples?

1024 total samples

− 60 × 17 packets = 1020 samples

--------------------------------

4 samples remaining

✔ This packet sends the final remainder

## 3️⃣ Final Packet – END (1 byte)

5a

### What it means:

- 0x5A → 'Z' = END / COMMIT packet

### What the MCU does here:

- Checks:

  - META packet was received

  - Exactly 1024 samples were received

- If valid → waveform is committed

- If not → upload is rejected

✔ This byte is always exactly 0x5A
✔ No payload, no length field

## Why the MCU Prints “NON-STANDARD PACKET”

The MCU firmware distinguishes between:

- Standard AWG packet ('S', 17 bytes)

- Arbitrary waveform packets ('A', 'D', 'Z')

So the log message:

>>> NON-STANDARD PACKET

Simply means:

“This is not the 17-byte AWG packet”

It does not indicate an error.

## Final Packet Flow Summary

| Order | Packet | Size | Purpose |
| --- | --- | --- | --- |
| 1 | 'A' | 6 bytes | Start upload (META) |
| 2 | 'D' | 123 bytes | 60 samples |
| 3 | 'D' | 123 bytes | 60 samples |
| … | 'D' | 123 bytes | More chunks |
| n | 'D' | 11 bytes | Final 4 samples |
| last | 'Z' | 1 byte | Commit waveform |

## Key Takeaways

- Multiple DATA packets are required

- Different packet sizes are normal

- The final 0x5A byte is mandatory

ARBITRARY FUNCTIONS- App sends full 64k Packet: (MCU Getting larger memory chip) PSRAM 8MG useable

- Expect transmission time to be longer

## ESP32 BLE Connection + Arbitrary Waveform Upload (Protocol)

### 1) BLE service + characteristics

The ESP32 advertises as ESP32-JOE and uses the Nordic UART Service (NUS).

Discover:

- Service UUID: 6e400001-b5a3-f393-e0a9-e50e24dcca9e

- RX (Write) characteristic UUID: 6e400002-b5a3-f393-e0a9-e50e24dcca9e ← app writes packets here

- TX (Notify) characteristic UUID: 6e400003-b5a3-f393-e0a9-e50e24dcca9e (optional; not required for upload)

Connection steps:

- Scan for ESP32-JOE

- Connect

- Discover NUS service

- Write all commands to the RX characteristic

### 2) Arbitrary waveform upload overview

Arbitrary waveform upload is 3 phases:

- META: start upload + set channel + set frequency

- DATA: stream 16-bit samples in chunks(Edit this so app sends all 64k points)

- END: commit (only succeeds if exactly 1024 samples received)

Total samples required: 1024 (exactly)

All multi-byte integers are Little Endian (LE).

### 3) Packets

#### A) META packet (start new ARB upload)

Format:
'A' + channel(1 byte) + freq_mHz(u32 LE)

- Byte 0: ASCII 'A' = 0x41

- Byte 1: channel:

  - 0 = Channel A

  - 1 = Channel B

- Bytes 2–5: freq_mHz as uint32 LE

  - Frequency is in milli-Hz (Hz * 1000)

Example: Channel A, 1000 Hz
freq_mHz = 1,000,000
Payload: [0x41, 0x00, 0x40, 0x42, 0x0F, 0x00]

Sending META resets/clears the ARB buffer on the MCU.

#### B) DATA packet (send samples)

Format:
'D' + count(u16 LE) + samples[count] (each sample u16 LE)

- Byte 0: ASCII 'D' = 0x44

- Bytes 1–2: count = number of samples in this packet (uint16 LE)

- Remaining bytes: count samples, each a uint16 LE DAC code

Repeat DATA packets until the MCU has received 1024 total samples.

#### C) END packet (commit)

Format:
'Z'

- Single byte: ASCII 'Z' = 0x5A

Commit rules:

- MCU only commits if META was received and exactly 1024 samples have been received total.

- If not 1024, it rejects the upload.

### 4) Sample encoding (what the 16-bit numbers mean)

Each sample is a raw uint16 DAC code: 0 … 65535.

If your app generates values in volts and the hardware range is ±5 V, map like this (clamp to range):

- code = round((v + 5.0) * (65535 / 10.0))

- v = −5.0 → 0

- v = +5.0 → 65535

(If the app already generates 0..65535 codes, send directly.)

### 5) BLE payload sizing / chunk size

DATA packet size is: 3 + 2*count bytes.

Pick count so the whole DATA packet fits in the BLE write payload (depends on MTU).

- If MTU is default (~20 payload bytes), max count ≈ 8 samples per packet.

- If MTU is negotiated larger (recommended), you can send bigger chunks (ex: 60 samples → 123 bytes).

Recommendation:

- Request a higher MTU if supported and stream DATA packets sequentially (write-with-response preferred for reliability).

### 6) Minimal send sequence summary

- Connect to BLE device ESP32-JOE

- Write META: 'A' + channel + freq_mHz(LE u32)

- Send DATA packets until 1024 samples total are sent

- Write END: 'Z'

MCU should…

Receive META 'A' with 10 bytes:

- [0]='A'

- [1]=channel

- [2..5]=freqMilliHz (uint32 LE)

- [6..9]=totalSamples (uint32 LE) → 32768

Receive many 'D' packets containing:

- 'D', count (uint16 LE), then count uint16 DAC samples

Stop on 0x5A