## Mobile App

### 1. Purpose

- The mobile app provides a simple, intuitive way to connect to the AWG and create, edit, and transmit user-defined waveforms.

- Built in Flutter for cross-platform support on iOS, Android, and PC.

- Main goal: give users an easy way to control waveform generation from a mobile device.

### 2. Design Requirements

- Sponsor requirements from the FSR contract:

  - Define offset, frequency, amplitude, waveform type, and channel

  - Maintain a reliable and stable BLE connection

  - Prioritize simplicity, ease of use, adjustability, and include user-safegaurds

- Built-in safeguards:

  - Prevents drawing invalid arbitrary waveforms such as intersecting values, backward traces, or values outside the amplitude range

  - Prevents entering frequency, VPP, or offset values outside allowed limits

  - Detects BLE disconnects and prompts the user to reconnect

  - Only searches for ESP32 BLE devices, making connection faster and clearer

### 3. Connection Screen

- This is the first screen users see.

- The app scans only for ESP32 BLE devices and displays their names for easy selection.

- Users can quickly connect to the AWG through BLE.

- If the connection is lost, the app alerts the user and prompts reconnection.

How it works

- BLE communication is handled through RX/TX between the app and the ESP32.

- This creates a simple wireless link between the mobile device and the AWG controller.

### 4. Standard Waveform Screen

- This screen functions like a traditional signal generator interface.

- Users can configure:

  - Frequency

  - Amplitude

  - Offset

  - Waveform type

  - Channel

How it works

- The app sends a parameterized string in a predefined format to the MCU.

- The MCU uses those parameters to select and generate standard waveforms from stored LUTs.

### 5. Drawable Arbitrary Waveform Screen

- This screen allows users to draw their own custom waveform on a canvas.

- The interface is designed to keep the drawing valid and within system limits.

- Users cannot draw non-functions or values outside the allowed amplitude range.

### 6. Editing Screen

- After drawing, users can refine the waveform before sending it.

- Users can apply edits such as:

  - Frequency

  - Channel

  - Offset

- This gives more control before transmitting the final waveform.

How it works

- The drawn waveform is converted into 64k discrete values.

- Those values are sent to the MCU along with the selected frequency, channel, and offset settings.

- The MCU then generates a unique LUT for that arbitrary waveform.

Enclosure

1. Purpose

- The enclosure houses and protects the system components while keeping the design compact, lightweight, and easy to service.

- It was designed to support the PCB securely while still allowing access for adjustments and future upgrades.

### 2. Design Requirements

- Required weight: less than 5.5 lbs

  - Final weight: 501 grams without the lid

- Required footprint: less than 12 x 8 x 10 inches

  - Final dimensions with lid: 8 x 8 x 5 inches

### 3. Structural Features

- The enclosure is self-supporting

- Uses Velcro for easy PCB removal and reinstallation

- Includes two open rail slots for interior PCB adjustability

- Extra interior space allows for:

  - wire routing

  - component adjustments

  - future upgrades if needed

### 4. Manufacturing and Material Choices

- The box was 3D printed in PLA and CADed using SOLIDWORKS

- Reduced interior infill

  - minimize weight

  - reduce material cost

  - maintain sufficient structural support