Need to set 2 values:

- Frequency Tuning word

  - 100/101 + 0x00000000

- Reset Value

  - 010/011 + 0x0000 + 0xXXXX

Max DAC Speed: 2 Megasamples / second

FPGA Clock Speed: 100 MHz

Digital Direct Synthesis

Frequency Tuning Word Equation: Fout=M*100E6 / 232

- M = binary tuning word

- Fout = Output frequency * Samples per period

Example:

- 10 Hz Wave

  - Fout = 10 Hz * 2^16 samples = 0.65536 MHz

  - Frequency tuning word: 0x01AD7F29

  - Reset value: 0xFFFF

- 15 Hz Wave

  - Fout = 15 Hz * 2^16 samples = 0.98304 MHz

  - Frequency tuning word: 0x02843EBE

  - Reset value: 0xFFFF

- 1 MHz / 2^16 = 15.2587891 Hz

  - Maximum frequency where whole lookup table can be used, assuming 1 MHz is maximum frequency

- 16 Hz Wave

  - Fout = 16 Hz * 2^16 samples = 1.048576 MHz

    - Higher than maximum frequency, set Fout at max frequency of 1 MHz

  - Frequency tuning word = (1MHz)/(100 MHz)(232) = 0x028F5C29

  - Samples = 1 MHz / 16 Hz = 62500 = 0xF424

- 100 kHz Wave

  - Frequency tuning word = 0x028F5C29

  - Samples = 1 MHz / 100 kHz = 10 = 0x000A

TODO:

- Arbitrary waveform integration

- Don’t reprogram memory if only changing the frequency

Pinout to MCU

(towards FPGA)

| MOSI (MCU -> FPGA) | Reset (Unused MISO) | SPI Clock (SCLK) |
| --- | --- | --- |
| VCC (Unused) | GND | Chip Select (CS) |

(outside of board)

Pinout to DAC

(towards FPGA)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | Clock |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | GND |

(outside of board)

Pinout on DAC Board

(outside of board)

| 11 | 12 | 13 | 14 | 15 | Clk |  | 5V |  | 0 | 1 | 2 | 3 | 3.3V | 4 | 5 | 6 | 7 | 8 | 9 | 10 | Gnd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(towards DAC)

Tables are seen with the top of the table pointing towards the FPGA.

The bottom of the DAC table is towards the edge of the board. Applies to both 18 pin headers.

All pins except the VCC on the 6 pin header should be used.

Colors on 18 pin table represent the groupings of wires to the analog board.

TA Email: balamurugan_22@tamu.edu